SM-A326U CARRIER UNLOCK — HANDOFF REPORT

Session Date: 24 March 2026  |  Status: IN PROGRESS — Awaiting Commercial Tool

Prepared for handoff. All prior work preserved. Commercial tool required to complete SIM unlock.

# 1. DEVICE IDENTITY

| Field | Value |
| --- | --- |
| Model | Samsung Galaxy A32 5G (SM-A326U) — US Variant |
| Serial | RFCR90NN63M |
| IMEI | 355820492174216 |
| Firmware | A326USQSHDYB2 (Android 13 / TP1A.220624.014.A326USQUADWG4) |
| Security Patch | 2025-02-01 |
| Chipset | MediaTek Dimensity 720 (MT6853V) — NOT Qualcomm |
| HW Rev | 1.0 |
| Storage | UFS — KM5P8001DM-B42 (Samsung), MID 0xCE |
| BROM HW Code | 0x996 / HW Subcode 0x8A00 |

# 2. ENVIRONMENT

| Item | Detail |
| --- | --- |
| PC | HP Pavilion x360 — Windows 10 Home, No admin (limited), PowerShell 5.1 |
| mtkclient | v2.1.4 (18 Mar 2026) — located at C:\Users\Source-1\Desktop\unlock\mtkclient |
| mtk.py path | C:\Users\Source-1\Desktop\unlock\mtkclient\mtk.py |
| Payload | mt6853_payload.bin — confirmed present in mtkclient\payloads\ |
| DA Loader | MTK_AllInOne_DA_V5.bin — auto-selected by tool (no --loader flag needed) |
| Driver | WinUSB via Zadig — MediaTek USB Port |
| USB | USB 2.0 port required — USB 3.0 causes detection failure |
| BROM Entry | Test-point method (CLK/CMD short) — confirmed reliable on this unit |

# 3. SESSION WORK COMPLETED

3.1 BROM Entry & Detection

- Zadig driver corrected from libusb0 to WinUSB

- BROM entry achieved via test-point (CLK/CMD short) with command-first timing method

- Kamakiri exploit executed successfully — mt6853_payload.bin loaded

- printgpt returned clean full GPT — all partitions confirmed present

- Connection stabilized — phone held BROM lock for remainder of session

3.2 Partitions Confirmed in GPT

- efs, nvram, nvdata, seccfg, protect1, protect2, carrier, boot, recovery, userdata — all present

3.3 Backups Taken (ALL SUCCESSFUL)

| File | Partition / Notes |
| --- | --- |
| efs.img | IMEI partition — CRITICAL |
| nvram.img | NVRAM |
| nvdata.img | Pre-seccfg baseline |
| protect1.img | Protect1 |
| protect2.img | Protect2 |
| seccfg.img | Pre-unlock seccfg baseline |
| carrier.img | Carrier partition |
| nvdata_post_seccfg.img | Post-seccfg nvdata — confirms clean write |

All files saved to: C:\Users\Source-1\Desktop\unlock\mtkclient\

3.4 seccfg Unlock

- Command: python mtk.py da seccfg unlock — SUCCESS

- Secure boot / seccfg layer disabled

- Post-seccfg nvdata dump confirmed clean

3.5 Carrier SIM Lock — NOT Resolved

- SIM lock test performed — phone remained carrier locked after seccfg unlock

- mtkclient manual NVRAM hex-edit path researched — no verified offset map exists for A326USQSHDYB2 on Android 13 as of March 2026

- protect1/protect2 erase option identified but not executed — opted for commercial tool instead

# 4. CRITICAL TECHNICAL NOTES

- MT6853 is NOT a V6 protocol chip — no --loader flag required

- No --payload flag needed — mt6853_payload.bin auto-loaded by tool

- Command-first timing required: run python mtk.py [cmd], THEN do test-point jump + plug in

- Never use USB 3.0 — detection fails

- After driver install: always verify Zadig shows WinUSB before running any command

- Max 2-3 failed attempts before Level 1 reset (driver purge + Device Manager cleanup)

- Ghost device issue: uninstall all greyed MTK entries EXCEPT active MediaTek USB Port during live session

# 5. NEXT STEPS — COMMERCIAL TOOL

The following tools have documented success on A326USQSHDYB2 + 2025-02-01 security patch:

| Tool | Notes |
| --- | --- |
| Cheetah Pro | Recommended — direct unlock on SPR/TMB/CHA carriers, BROM-aware |
| Q-UnLocker | Verified on identical firmware/baseband |
| ChimeraTool | Verified — handles DA/preloader internally |
| Global Unlocker Pro | Documented success on this build |
| Tira Unlocker v9.4+ | Works without test-point in latest versions |

The commercial tool will use the same BROM/test-point entry method. All backups are in place. seccfg is already unlocked — inform the tool operator of this.

# 6. RECOVERY REFERENCE

If anything goes wrong during commercial tool operation:

- EFS (IMEI) restore: python mtk.py w efs efs.img

- NVRAM restore: python mtk.py w nvram nvram.img

- nvdata restore: python mtk.py w nvdata nvdata.img

- protect1/2 restore: python mtk.py w protect1 protect1.img / protect2 protect2.img

- seccfg restore: python mtk.py w seccfg seccfg.img

- Full preloader restore: python mtk.py rl preloader (uses backup preloader.bin)

All recovery commands require BROM entry first (test-point + command-first timing).

# 7. KNOWN ISSUES / FORBIDDEN PATTERNS

- FP: Never use python mtk.py print — command is invalid in v2.1.4, use printgpt

- FP: libusb0 / libusb-win32 driver causes timeouts — WinUSB only

- FP: Running command AFTER connecting causes handshake timeout — always command-first

- FP: USB 3.0 port causes detection failure — USB 2.0 only

- FP: Retrying >3 times without driver reset creates ghost device accumulation

Report generated: 24 March 2026 | mtkclient v2.1.4 | SM-A326U / MT6853 / A326USQSHDYB2